CAN-7USAT

ATMOSPHERIC POLLUTION PROFILING CANSAT

Version 5.0 — Air-Quality Payload + GPS-Guided Targeted Landing

IN-SPACe CAN-7USAT India Student Competition 2026 (3rd Edition)

SCIENCE MISSION Build a vertical pollution profile of the atmospheric column from 1000 m to ground: particulate matter (PM1.0/2.5/10), traffic gases (NO2/CO), combustion CO2, industrial/agricultural off-gassing (NH3/benzene), VOC index and UV - a wildfire-smoke and urban-air-quality mapping payload.

Retains full compliance with Doc. IN-SPACe.CAN.MR.CG.01 (3rd Ed. 2026): two-stage descent, mechanical momentum-wheel stabilization, exact telemetry, all 42 requirements (Section 16). PLUS claims optional objectives (i) trajectory control, (iii) full-flight video capture, and (iv) targeted landing - alongside (ii) the sensor payload.

Bennett University — Astronomy Club

Space Tech Division

| Document Version | 5.0 (Payload + Targeted Landing Edition) |
| --- | --- |
| Descent | Drogue (20 m/s) + GPS-guided steerable main (1-3 m/s) |
| Stabilization | Brass momentum wheel (mechanical gyro) |
| Science Payload | BME688 + MiCS-4514 + SPS30 + SCD41 + MQ-135 + GUVA UV |
| Imaging | Full-flight video (separation -> touchdown) to onboard microSD |
| Timekeeping | DS3231 battery-backed RTC + GNSS time + flash-persisted clock |
| Mass / Envelope | ~710g / 148mm dia x ~390mm (within 1kg / 150x400mm) |
| Build Cost | ~Rs. 46,000 (within Rs. 1,00,000 cap) |

# 1. MISSION OVERVIEW

This CAN-7USAT carries a dedicated atmospheric pollution and wildfire-smoke profiling payload. During descent from 1000 m, it builds a vertical profile of particulate matter, traffic-exhaust gases, combustion CO2, industrial/agricultural off-gassing, VOCs and UV irradiance over the launch site. The platform retains all mandatory competition functions: two-stage parachute descent, mechanical gyroscopic stabilization, real-time telemetry, onboard logging, full-flight onboard video, and recovery via a 95 dB audio beacon and power-indicator light.

## 1.1 Mission Objectives

| Primary (mandatory) | Real-time telemetry of position, altitude, pressure, temperature, orientation, power, system status; two-stage descent control |
| --- | --- |
| Mechanical Gyro (mandatory) | Momentum-wheel attitude stabilization with spin-rate telemetry |
| Science Payload (bonus ii) | Vertical pollution profile: PM1.0/2.5/10, NO2, CO, CO2, NH3/benzene, VOC index, UV |
| Trajectory Control (bonus i) | GPS-guided steerable main canopy = novel descent trajectory control |
| Video Capture (bonus iii) | Onboard camera records separation -> touchdown to its own microSD |
| Targeted Landing (bonus iv) | Homes toward stored launch-pad GPS waypoint; flares for soft touchdown |
| Application | Urban air-quality mapping + wildfire smoke-column detection |
| Recovery | 95 dB beacon (>=92 dB req) + indicator light + tri-lingual labeling + GNSS coordinates |

WHY THIS WINS POINTS This entry claims FOUR optional objectives: (i) novel descent trajectory control, (ii) additional innovative sensors (6-sensor atmospheric-chemistry payload), (iii) video capture from separation to touchdown, and (iv) controlled flight & targeted landing (GPS-guided steerable parachute). Each earns extra weightage per Rule 3.2, on top of a fully compliant, flight-proven base platform.

# 2. ATMOSPHERIC SCIENCE PAYLOAD

Six sensors form the payload, grouped by what they detect. Each is justified by a specific pollution or combustion tracer.

## 2.1 Payload Sensor Suite

| Sensor | Type / Interface | Measures | Pollution / Science Role | Mass |
| --- | --- | --- | --- | --- |
| BME688 | Digital I2C | VOCs, sulfur compounds, humidity, T, P | Broad air-quality baseline + smog indicator (IAQ index) | ~2 g |
| MiCS-4514 | Analog (2 outputs) | NO2 + CO | Definitive tracer for vehicle exhaust / urban traffic | ~5 g |
| Sensirion SPS30 | Digital I2C/UART | PM1.0, PM2.5, PM10 (laser) | Urban haze/smog + wildfire ash & soot columns | ~26 g |
| SCD41 | Digital I2C | CO2 (photoacoustic, true) | Forest-fire CO2 spikes + urban greenhouse gradient | ~5 g |
| MQ-135 | Analog | Ammonia (NH3), benzene | Industrial pollution + agricultural off-gassing | ~15 g |
| GUVA-S12SD | Analog | UV (200-400 nm) | UV irradiance vs altitude (atmospheric absorption) | ~1 g |

Recommended choices within the options: SPS30 over PMS5003 (lighter 26g vs 40g, I2C, higher build quality); SCD41 over SCD40 (newer, smaller, lower power); BME688 over BME680 (improved gas sensing + AI classification via Bosch BSEC).

## 2.2 The Vertical Profile Concept

ALTITUDE   WHAT THE PAYLOAD MAPS DURING DESCENT

1000m  ___ Clean upper air: baseline VOC, CO2, low PM

|

600m  ___ Main chute opens; slow 1-3 m/s = dense sampling

|   PM2.5 rises entering haze/smog layer

|   NO2/CO climb near traffic corridors

|   CO2 gradient over urban/combustion sources

200m  ___ NH3/benzene from industrial/agri sources

|   Peak particulate near surface (soot/ash)

0m  ___ Surface concentrations logged at touchdown

Result: a stacked vertical chemistry profile of the

air column - the core scientific deliverable.

HONEST LIMITATIONS (state these at PDR/CDR) MOS sensors (MQ-135, MiCS-4514) give RELATIVE concentration trends, not lab-grade ppm; they need 24-48h burn-in + calibration and have cross-sensitivity. Descent speed limits vertical resolution (SCD41 samples ~every 5s = ~5-15m per reading at 1-3 m/s). The PM sensor needs clean ducted airflow; ram-air and orientation add uncertainty. Report data as relative vertical profiles, with calibration curves shown.

THERMAL ISOLATION (critical) MQ-135 and MiCS-4514 have heaters that generate heat. They are placed in a separate, vented Heated-Gas Bay, thermally isolated from the precision environmental sensors (BMP390, BME688) and the temperature-sensitive SCD41, to prevent skewing their readings.

# 3. PHYSICAL DESIGN & LAYOUT

## 3.1 Compliance Envelope

| Max diameter | 150 mm -> Design: 148 mm |
| --- | --- |
| Max height | 400 mm -> Design: ~385 mm (15mm margin) |
| Max mass | 1000g +/-50g -> Design: ~710g (290g margin) |
| Body color | Fluorescent orange + Indian flag (mandatory) |
| Edges | All radiused >= 2mm |
| Loads | 15 G sustained + 30 G shock |
| RF transparency | Fiberglass shell (RF-transparent) |

## 3.2 Cross-Section Layout (7 bays, top to bottom)

___________________________

/                           \   GNSS patch antenna (zenith)

|   TOP CAP + GNSS ANTENNA    |   15mm   fluorescent + flag

|___________________________ |

|  PRIMARY DROGUE BAY         |   40mm   drag-deployed,

|  (pre-attached, no mech)    |          pre-tied to body

|===========================  | -- Ring 1

|  STEERABLE MAIN BAY         |   65mm   GPS-guided cruciform;

|  [release + 2 winch servos] |          servo-steered @600m

|===========================  | -- Ring 2

|  MOMENTUM WHEEL (gyro)      |   40mm   brass wheel + motor

|    ()==[MOTOR]==()  [Hall]  |          + 2 bearings + Hall

|===========================  | -- Ring 3

|  AVIONICS + CLEAN-AIR ENV   |   45mm   ESP32 + custom PCB;

|  ESP32|BMP390|BME688|RTC    |          BMP390, BME688, BNO055,

|  BNO055|SCD41|DS3231|uSD    |          SCD41, DS3231 RTC (I2C).

|  [mesh vent]                |          Mesh-screened vent.

|===========================  | -- Ring 4

|  PARTICULATE BAY            |   30mm   SPS30 flat-mounted;

|  inlet-> [SPS30 laser] ->out|          ducted clean airflow

|===========================  | -- Ring 5

|  HEATED-GAS BAY (isolated)  |   35mm   MQ-135 + MiCS-4514

|  [MQ135][MiCS4514][GUVA UV] |          + UV; thermally

|  vented; away from BMP/SCD  |          isolated; heater MOSFET

|===========================  | -- Ring 6

|  COMMS + CAMERA BAY         |   35mm   XBee Pro S2C + whip;

|  [XBee][NavIC][CAMERA]->win |          NavIC rx; action cam

|                             |          (lens at side window)

|===========================  | -- Ring 7

|  POWER BAY                  |   70mm   18650 (solder-tab) sled;

|  [18650][3.3V][5V 3A boost] |          3.3V LDO + 3A 5V boost;

|  [95dB buzzer][SWITCH+LED]  |          beacon; ext switch+LED

|___________________________ |

|     BOTTOM CAP + bumper     |   15mm

\___________________________/

Diameter 148mm | Height ~390mm | Mass ~710g

AIRFLOW DESIGN: The particulate bay has a dedicated air inlet (top) and outlet (bottom) so the SPS30's internal fan draws a clean sample stream. Gas-sensor bays use mesh-screened vents. All vents are sensor openings (permitted); the rest of the electronics stays enclosed per Rule 5.2-i.

ENTANGLEMENT MITIGATION (CDR 12-f): Drogue and main share the top attachment via a fishing swivel; main lines are longer so it inflates above the trailing drogue. Standard, proven.

# 4. DESCENT, RECOVERY & TARGETED LANDING

## 4.1 Two Descent Mechanisms + Gyro Stabilizer

| Stage 1 — Drogue | Cruciform, ~25cm span, Cd~0.8. PASSIVE drag-deploy at ejection, pre-attached to body, NO mechanism. ~20 m/s (Rule 5.4-iii,iv,22) |
| --- | --- |
| Stage 2 — Steerable Main | Cruciform, ~2.0m span, Cd~0.8. Servo-pin release at 600m (+/-10m). ~2.5 m/s (within 1-3 m/s, Rule 23). GPS-steered toward launch pad. |
| Steering | 2x winch servos (GPIO14/12) pull L/R control lines on adjacent trailing edges; ~20cm stroke = full turn; both = flare for soft landing |
| Gyro stabilizer | Brass momentum wheel 45mm x 5mm, ~50g, on yaw axis, coreless motor (PWM), 6-10k RPM, A3144 Hall RPM. Spins up at ROCKET_DEPLOY (Rule 5.4-viii, 25) |
| No pyrotechnics | Servo/mechanical only (Rule 5.4-vii, 24) |

## 4.2 GPS Guidance (optional objectives i + iv)

Before launch the launch-pad coordinate is stored. During guided descent the ESP32 (5-10 Hz) computes the great-circle bearing to the pad, compares it with heading (GPS course, or BNO055 compass at low speed), and pulls the appropriate control line proportionally (u = Kp x heading_error). Within a capture radius it loiters; below ~15 m it flares. Fail-safe: GNSS loss or low battery centers both servos -> plain compliant 1-3 m/s descent. (Concept adapted & cited from open-source R2Home; own ESP32 implementation. Full detail in the Parachute & Descent Subsystem document.)

## 4.3 Descent Sequence with Payload + Guidance

~1000m  EJECT -> drogue drag-deploys; wheel spins up   [State 4]

Payload gas heaters already warm (pre-launch)

1000->600m  Descent ~20 m/s; full telemetry + payload  [State 5]

600m  Release MAIN -> ~2.5 m/s; GPS guidance ACTIVE  [State 6]

600->15m  GUIDED descent: steer toward launch pad      [State 6b]

+ DENSE pollution sampling (slow descent)

~15m  FLARE (pull both lines) for soft landing

0m  TOUCHDOWN -> 92dB beacon; surface readings; GPS  [State 7]

ENTANGLEMENT MITIGATION (CDR 12-f): drogue + steerable main share one swivel; main lines are longer so it inflates above the small trailing drogue; control lines routed through guide rings. Drogue stays attached and compliant.

# 5. SENSOR & DATA SUBSYSTEM (full sensor list)

## 5.1 Mandatory + Payload Sensors

| Function | Sensor | Bus / Pin | I2C addr | Notes |
| --- | --- | --- | --- | --- |
| Altitude/Pressure/Temp (mandatory) | BMP390 | I2C | 0x77 | Primary altimeter (0.1m) |
| Orientation/Accel (mandatory) | BNO055 | I2C | 0x28 | On-chip fused orientation |
| Gyro spin rate (mandatory) | A3144 Hall | GPIO27 | - | Momentum-wheel RPM -> deg/s |
| Position (mandatory, +pts) | NavIC GNSS | UART2 16/17 | - | NavIC = extra weightage |
| Power (mandatory) | Divider -> ADC | GPIO35 | - | Bus voltage 0.01V |
| VOC/humidity/air-quality | BME688 | I2C | 0x76 | BSEC IAQ index |
| CO2 | SCD41 | I2C | 0x62 | Photoacoustic; periodic mode |
| PM1.0/2.5/10 | SPS30 | I2C | 0x69 | Laser; fan-drawn sample |
| NO2 + CO | MiCS-4514 | GPIO33 + GPIO36 | - | 2 analog outputs (heated) |
| NH3 + benzene | MQ-135 | GPIO32 | - | Analog (heated) |
| UV irradiance | GUVA-S12SD | GPIO34 | - | Analog |
| Real-time clock | DS3231 RTC | I2C | 0x68 | Battery-backed wall-clock; SD + telemetry timestamps |
| Imaging (optional iii) | Action cam (RunCam Thumb) | 5V via GPIO15 | - | Records full flight to own microSD |

Six I2C devices share one bus (GPIO21/22) with distinct addresses (0x77, 0x76, 0x28, 0x62, 0x69, 0x68) - no conflict. Five analog signals occupy ESP32 ADC1 (GPIO 32/33/34/35/36). A heater-enable MOSFET (GPIO13) powers the MQ-135 + MiCS heaters only when needed; two winch servos (GPIO14/12) steer the main canopy. A switched 5V line (GPIO15 MOSFET) powers the full-flight camera, which records to its own microSD. The DS3231 RTC has a CR2032 backup cell so accurate time survives power-off.

# 6. ELECTRICAL POWER SUBSYSTEM

| Cell | 18650 Li-ion, 3.7V, 3000-3500 mAh (metal can; LiPo prohibited). 3500mAh (e.g. NCR18650GA) recommended for payload margin |
| --- | --- |
| Connections | Soldered tabs / spot-welded nickel - NO spring contacts (Rule 5.3-v) |
| Battery access | Removable screw-in sled; swap < 1 min (Rule 5.3-iv) |
| Regulation | AMS1117-3.3 (3.3V) + 3A boost (5V for gyro motor, SPS30 fan, gas heaters, servo) |
| External switch | Toggle WITH indicator LED + power-on buzzer chirp (Rule 5.3-i: light OR sound - both provided) |
| Audio beacon | 95 dB(A) siren buzzer (exceeds 92 dB min, Rule 5.2-v); continuous after landing, can also sound on pad |
| Camera power | Action cam on switched 5V (GPIO15 MOSFET) OR own internal cell; enabled pre-launch |
| Heater management | MQ-135 + MiCS heaters gated by MOSFET; powered ~30 min pre-launch via RF command (not full 2h pad wait) |

## 6.1 Power Budget

| Component | Rail | mA | When | Notes |
| --- | --- | --- | --- | --- |
| ESP32 | 3.3V | 240 | always | Flight computer |
| BMP390 | 3.3V | 1 | always | Altimeter |
| BNO055 | 3.3V | 12 | always | IMU |
| BME688 | 3.3V | 15 | always | VOC/humidity |
| SCD41 | 3.3V | 18 | always | CO2 (avg; 205 peak) |
| NavIC GNSS | 3.3V | 50 | always | Tracking |
| XBee Pro S2C | 3.3V | 120 | always | Telemetry |
| MicroSD | 3.3V | 40 | ~50% | Write bursts |
| GUVA UV + Hall | 3.3V | 5 | always | Analog + RPM |
| SPS30 (fan) | 5V | 60 | sampling | PM laser+fan |
| MiCS-4514 heater | 5V | 80 | warmed | Pre-launch +flight |
| MQ-135 heater | 5V | 150 | warmed | Pre-launch +flight |
| Gyro motor | 5V | 150 | descent | Momentum wheel |
| Steering servos x2 | 5V | 150 | descent | Canopy guidance (pulsed avg) |
| Release servo | 5V | 250 | <1% | Single @600m |
| DS3231 RTC | 3.3V | 2 | always | uA in backup (CR2032) |
| Action camera | 5V | 200 | from pre-launch | Records to own microSD (or own cell) |
| Camera | 5V | 200 | CAM_ON->land | RunCam-class; own microSD |
| Beacon 95dB | 5V | 60 | post-land | Recovery (>=92 dB) |

Pad phase (heaters/servos off): ~500 mA. Last 30 min + flight (heaters + fan + gyro + steering on): ~1.2 A. Energy for 2h pad + flight ~= 1.55 Ah. A 3500 mAh cell covers this with margin (recommended for the steering + payload load).

# 7. AVIONICS PCB & PIN MAP

| GPIO | Function | Bus | Connected To |
| --- | --- | --- | --- |
| 21 / 22 | I2C SDA / SCL | I2C | BMP390 + BME688 + BNO055 + SCD41 + SPS30 + DS3231 RTC (4.7k pull-ups) |
| 16 / 17 | UART2 RX / TX | UART | NavIC GNSS |
| 1 / 3 | UART0 TX / RX | UART | XBee Pro S2C |
| 5/18/19/23 | CS/SCK/MISO/MOSI | SPI | MicroSD |
| 32 | MQ-135 analog | ADC1 | NH3/benzene |
| 33 | MiCS-4514 CO | ADC1 | CO output |
| 36 (VP) | MiCS-4514 NO2 | ADC1 | NO2 output |
| 34 | GUVA UV analog | ADC1 | UV |
| 35 | Battery voltage | ADC1 | Divider (2x100k) |
| 13 | Gas heater enable | Digital | MOSFET -> MQ135+MiCS heaters |
| 15 | Camera power enable | Digital | MOSFET -> camera 5V (CAM_ON) |
| 25 | Release servo PWM | PWM | Main-parachute release pin |
| 14 | Steering servo LEFT | PWM | Left winch servo (canopy) |
| 12 | Steering servo RIGHT | PWM | Right winch servo (canopy) |
| 26 | Gyro motor PWM | PWM | MOSFET -> coreless motor |
| 27 | Hall input | Digital | A3144 RPM (interrupt) |
| 4 | Beacon | Digital | 95dB buzzer (transistor) |
| 2 | Status LED | Digital | Switch indicator |

- 2-layer FR4 PCB ~90x65mm; continuous bottom ground plane; JLCPCB (~Rs.600).

- Hard-mounted with M3 standoffs (Rule 5.2-iii); no breadboard/spring connections.

- Separate analog ground for MQ-135/MiCS to reduce heater-noise on ADC readings.

- Bulk 470uF + 100nF on the 5V rail near motor/fan/heaters; star ground at battery.

# 8. FLIGHT SOFTWARE

## 8.1 State Machine (official states, Rule 6.2)

| State | ID | Trigger | Actions (incl. payload) |
| --- | --- | --- | --- |
| BOOT | 0 | Power on | Init all sensors + SD + GNSS + RTC; self-test; restore time/state if reset |
| TEST_MODE | 1 | Ground cmd | Simulation; accept SIM altitude; verify transitions |
| LAUNCH_PAD | 2 | ARM cmd | TX after RF cmd; CAL zeroing; store pad GPS waypoint; HEATER_ON + CAM_ON ~30min pre-launch |
| ASCENT | 3 | Accel>3G | Boost; log; payload sampling continues |
| ROCKET_DEPLOY | 4 | Ejection | Drogue via drag; spin up wheel; steering servos centered |
| DESCENT | 5 | Under drogue | Descent ~20 m/s; full telemetry + payload + SD; arm guidance |
| AEROBREAK_RELEASE | 6 | Alt<=600m | Release steerable main; descent -> ~2.5 m/s; guidance ON |
| GUIDED_DESCENT | 6b | Main open | Steer toward pad waypoint; dense sampling; loiter in capture radius |
| IMPACT | 7 | Landed | Flare->touchdown; stop motor; beacon ON; GPS TX; finalize CSV |

RESET SAFETY: mission time + state persisted to ESP32 flash (NVS) each packet, and a DS3231 battery-backed RTC holds absolute wall-clock time across power-off. On reset, elapsed mission time is reconstructed so the clock never restarts (Rule 6.1-ix, 6.2-ii).

## 8.2 Telemetry Format (16 mandatory + appended payload fields)

Rule 6.3 allows appending fields after the 16 required. Filename Flight_<TEAM_ID>.csv, ASCII CSV + carriage return, >=1 Hz.

| # | Field | Unit | Source |
| --- | --- | --- | --- |
| 1-3 | TEAM_ID, TIME_STAMPING, PACKET_COUNT | -, s, int | System |
| 4 | ALTITUDE | 0.1 m | BMP390 |
| 5-6 | PRESSURE, TEMP | Pa, 0.1C | BMP390 |
| 7 | VOLTAGE | 0.01 V | ADC |
| 8-12 | GNSS TIME/LAT/LON/ALT/SATS | s, deg, m, int | NavIC |
| 13 | ACCELEROMETER_DATA | m/s2 | BNO055 |
| 14 | GYRO_SPIN_RATE | deg/s | A3144 (wheel) |
| 15 | FLIGHT_SOFTWARE_STATE | 0-7 | State machine |
| 16 | OPTIONAL_DATA (flag) | - | Payload/guidance status |
| 17-19 | PM1_0, PM2_5, PM10 | ug/m3 | SPS30 |
| 20-21 | NO2, CO | ppb/ppm (rel) | MiCS-4514 |
| 22 | CO2 | ppm | SCD41 |
| 23 | NH3_BENZENE | rel | MQ-135 |
| 24-25 | VOC_INDEX, HUMIDITY | IAQ, %RH | BME688 |
| 26 | UV_INDEX | index | GUVA |
| 27-29 | BEARING, DIST_TO_PAD, HEADING | deg, m, deg | Guidance |
| 30 | UTC_TIME | hh:mm:ss | DS3231 RTC |

# 9. GROUND STATION

| Platform | Laptop (>=2h battery) + Python GUI (PyQt5 + pyqtgraph) |
| --- | --- |
| Radio | XBee Pro S2C on USB explorer; Yagi on tripod, elevated |
| Display | All 16 mandatory + payload + guidance fields plotted in real-time (SI units) |
| Pollution dashboard | Live vertical-profile plots: PM/NO2/CO/CO2/NH3/VOC/UV vs altitude |
| Guidance display | Live map: CANSAT track, launch-pad waypoint, bearing/distance-to-pad |
| Commands | START_TX, STOP_TX, CAL, SET_WAYPOINT, HEATER_ON/OFF, SIM_ENABLE/ACTIVATE |
| Logging | Auto Flight_<TEAM_ID>.csv with headers + RTC UTC timestamps (Rule 6.3-ii) |
| Map + portability | Live GNSS track; mobile for recovery |

# 10. COMPLETE BILL OF MATERIALS

## 10.1 Core Electronics (mandatory systems)

| Item | Model | Qty | Rs. | Origin |
| --- | --- | --- | --- | --- |
| Flight computer | ESP32-WROOM-32 | 1 | 500 | Import |
| Barometer | BMP390 | 1 | 900 | Import |
| Fused IMU | BNO055 | 1 | 1,400 | Import |
| GNSS (NavIC) | NavIC rx + antenna | 1 | 3,500 | India (pref.) |
| Hall sensor | A3144 + magnet | 1 | 40 | India |
| Telemetry pair | XBee Pro S2C x2 + antennas | 2 | 4,500 | Import |
| XBee USB explorer | Ground adapter | 1 | 500 | Import |
| MicroSD + card | SPI + 16GB | 1 | 400 | India |
| RTC module | DS3231 + CR2032 | 1 | 150 | India |
| Action camera | RunCam Thumb (4K, microSD) | 1 | 2,800 | Import |
| Camera mount + window | PC window + bracket | 1 | 200 | India |
| 18650 cell | Li-ion 3500mAh (metal can) | 1 | 450 | India |
| Battery sled | Spot-weld nickel + retainer | 1 | 120 | India |
| Regulators | AMS1117-3.3 + 3A boost | 2 | 150 | India |
| Beacon + switch | 95dB buzzer + toggle + LED | 1 | 160 | India |
| Custom PCB | 2-layer FR4 (5 pcs) | 1 | 600 | India |
| Connectors/passives | JST, headers, R/C, MOSFETs | set | 500 | India |

Core electronics subtotal: ~Rs. 16,870 (incl. DS3231 RTC + action camera).

## 10.2 Atmospheric Payload Sensors

| Item | Measures | Qty | Rs. | Origin |
| --- | --- | --- | --- | --- |
| BME688 breakout | VOC/humidity/T/P | 1 | 1,200 | Import |
| MiCS-4514 | NO2 + CO | 1 | 1,500 | Import |
| Sensirion SPS30 | PM1.0/2.5/10 | 1 | 4,000 | Import |
| SCD41 | CO2 | 1 | 2,500 | Import |
| MQ-135 | NH3/benzene | 1 | 250 | Import |
| GUVA-S12SD | UV | 1 | 250 | Import |
| Payload mount + ducting | Airflow + thermal isolation | set | 350 | India |
| (Alt: PMS5003) | cheaper PM, heavier 40g | (alt) | 1,200 | Import |

Payload subtotal (with SPS30): ~Rs. 10,050. With PMS5003 instead: ~Rs. 7,250.

## 10.3 Mechanical, Descent & Structure

| Item | Spec | Qty | Rs. | Origin |
| --- | --- | --- | --- | --- |
| Brass momentum wheel | 45mm x 5mm machined | 1 | 300 | India |
| Coreless DC motor | 716/N20-class | 1 | 250 | India |
| Wheel bearings | MR105ZZ | 2 | 80 | India |
| Main parachute | Cruciform Nylon ~2.0m steerable | 1 | 900 | India |
| Drogue parachute | Cruciform Nylon ~25cm | 1 | 300 | India |
| Lines + swivel | Nylon cord + swivel | set | 200 | India |
| Release servo + pin | SG90 + steel pin + spring | set | 300 | Import/India |
| Steering winch servos | 2x multi-turn winch servo + spools | 2 | 1,400 | RC supplier |
| Control lines + guide rings | Dacron + stainless rings | set | 250 | India |
| Fiberglass + foam + epoxy | E-Glass 1kg, foam, resin | set | 3,400 | India |
| PLA+ filament | 1.75mm orange | 1 | 1,398 | India |
| Layup consumables | Peel ply, film, breather, PPE | set | 4,000 | India |
| Flag decal + paint + fasteners | Fluorescent + flag + M3 | set | 600 | India |

Mechanical + structure + steering subtotal: ~Rs. 13,378.

## 10.4 Budget Summary

| Category | Rs. | % of cap |
| --- | --- | --- |
| Core electronics (RTC + camera) | 16,870 | 16.9% |
| Atmospheric payload | 10,050 | 10.1% |
| Mechanical + structure + steering | 13,378 | 13.4% |
| Contingency + spares (15%) | 6,044 | 6.0% |
| TOTAL BUILD | 46,342 | 46.3% |
| Registration + CDR fees | 25,000 | — |

Build ~Rs. 46k, well under the Rs. 1,00,000 cap. Sponsors may fund up to Rs. 2,00,000 (excl. travel). Core structure/PCB/battery/mechanics/steering are India-sourced; payload sensors are imports (science-bonus tradeoff).

# 11. MASS BUDGET

| Subsystem | Items | Mass (g) |
| --- | --- | --- |
| Avionics | ESP32 + PCB + BMP390 + BNO055 + standoffs | 42 |
| GNSS | NavIC rx + antenna | 20 |
| Comms | XBee Pro S2C + whip | 12 |
| Storage + RPM + RTC | MicroSD + Hall + DS3231 | 11 |
| PAYLOAD: BME688 | VOC/humidity | 2 |
| PAYLOAD: MiCS-4514 | NO2/CO | 5 |
| PAYLOAD: SPS30 | PM laser sensor | 26 |
| PAYLOAD: SCD41 | CO2 | 5 |
| PAYLOAD: MQ-135 | NH3/benzene | 15 |
| PAYLOAD: GUVA UV | UV | 1 |
| Payload mount + ducting | Airflow + isolation | 15 |
| Momentum wheel | Brass + motor + bearings + mount | 80 |
| Drogue + steerable main | Cruciform chutes + lines + swivel | 74 |
| Steering + release | 2 winch servos + release servo + spools | 74 |
| Power | 18650 + sled + regs (3A boost) | 75 |
| Beacon + switch + light | 95dB buzzer + toggle + indicator LED | 10 |
| Camera + mount + window | Action cam (RunCam Thumb) + bracket | 14 |
| Structure | Shell + caps + 7 bulkheads | 190 |
| Brackets + wiring + fasteners | PLA mounts + cabling | 55 |

TOTAL: ~710g (with SPS30 + camera). Margin: ~290g under the 1000g limit. Use spare margin to trim/ballast for CG.

# 12. TEST & VERIFICATION PLAN

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Sensor bench | Power up, read all 11 sensors 10 min | Valid data, no I2C/UART/ADC errors |
| Gas calibration | Burn-in MQ-135/MiCS 24-48h; expose to known sources | Repeatable relative response curves |
| PM airflow | Run SPS30 with ducting; smoke source test | PM2.5 rises with smoke; stable fan flow |
| CO2 check | Breathe near SCD41 / known CO2 | Reads ~400ppm ambient, rises on exposure |
| Thermal isolation | Run heaters; monitor BMP390/SCD41 temp | Precision sensor drift < 1C from heaters |
| Power (heater-managed) | 2h pad (heaters off) + 30min warm + flight load | Endurance met on single 18650 |
| Drop test | 2m drop on chute cord | No damage, telemetry continues |
| Thermal soak | 38-40C, 2h, all on | All sensors nominal |
| Vibration | 60s launch-profile shake | No detachment (15G/30G intent) |
| Parachute deploy | Release x10; drogue drag x5 | Reliable deploy, no entanglement |
| Steering authority | Manual L/R during drone drop | Coordinated turns; ~20cm pull = full turn |
| GPS guidance | Drone drop toward marked pad | Homes toward waypoint; lands near target |
| Guidance fail-safe | Cut GNSS mid-descent | Servos center; straight stable descent |
| RTC timekeeping | Power-cycle; compare RTC vs GNSS time | Accurate UTC retained across power-off |
| Camera recording | CAM_ON; full flight-duration record test | Continuous video separation->touchdown on microSD |
| Beacon SPL | Sound-level meter at 1m | >=92 dB(A) (target 95 dB) |
| Gyro wheel | Spin-up + Hall RPM; tilt body | Stable RPM telemetry; attitude stiffness |
| Full mission sim | SIM altitude profile via GS | All 9 states; full CSV (mandatory+payload+guidance) |
| Comm range | 1.5km LOS | >95% packets |

# 13. COMPETITION TIMELINE (OFFICIAL)

| Phase | Activity | Window |
| --- | --- | --- |
| Register | Apply + Rs.10,000 on IDP | by 30 Apr 2026 |
| Screening | Evaluation & selection | 01-15 May 2026 |
| PDR submit | Preliminary Design Review doc | 16-30 May 2026 |
| PDR review | Jury presentation (10 min) | 01-30 Jun 2026 |
| CDR | Comprehensive Design Review + Rs.15,000 | 01-31 Jul 2026 |
| CDR complete | Jury completion | 01-31 Aug 2026 |
| FRR | Flight Readiness Review | 01 Sep-15 Oct 2026 |
| Launch | Final + results (Kushinagar) | 16 Oct-30 Nov 2026 |

ACT NOW Register by 30 April 2026 (tomorrow) - Rs.10,000 to SSDF (HDFC A/c 50200118890683, IFSC HDFC0010007) on www.inspace.gov.in. Order the SPS30/SCD41/MiCS early - imported sensors have long lead times. Begin MOS sensor burn-in immediately on arrival (24-48h).

# 14. REQUIREMENTS COMPLIANCE MATRIX

All 42 requirements remain compliant; payload and steering are additive and include fail-safes that preserve mandatory functions. Three optional objectives are claimed at the end.

| # | Requirement (abridged) | Status | How met |
| --- | --- | --- | --- |
| 1 | Mass < 1000g +/-50g | Compliant | ~695g |
| 2 | Fit 0.15m x 0.40m | Compliant | 148mm x 385mm |
| 3 | No sharp edges | Compliant | Radiused >=2mm |
| 4 | Fluorescent + flag | Compliant | Orange + Indian flag |
| 5 | No airframe use | Compliant | Free-flying |
| 6 | Mandatory datasets | Compliant | BMP390+BNO055+GNSS+Hall+ADC |
| 7 | Real-time GS display | Compliant | PyQt GUI all fields |
| 8 | Onboard SD backup | Compliant | MicroSD CSV + RTC timestamps |
| 9 | Electronics enclosed | Compliant | Shell; only sensors vented |
| 10 | 15G / 30G | Compliant | FRP + hard mounts |
| 11 | Hard-mounted PCB | Compliant | M3 standoffs |
| 12 | Tri-lingual labels | Compliant | Eng+Hindi+regional |
| 13 | Beacon >=92 dB | Compliant | 95 dB |
| 14 | Switch + indicator | Compliant | Toggle + LED |
| 15 | 2h pad battery | Compliant | Heater-managed, 3500mAh |
| 16 | Approved battery | Compliant | 18650 metal can |
| 17 | Battery <1 min swap | Compliant | Screw-in sled |
| 18 | No spring contacts | Compliant | Soldered tabs |
| 19 | Two descent mechs | Compliant | Drogue + steerable main |
| 20 | Primary at ejection | Compliant | Drogue drag-deploy |
| 21 | Primary attached, no mech | Compliant | Pre-tied, passive |
| 22 | Primary 20 m/s +/-5 | Compliant | ~25cm cruciform drogue |
| 23 | Secondary @600m 1-3 m/s | Compliant | ~2.0m steerable main, 2.5 m/s |
| 24 | No pyrotechnics | Compliant | Servo only |
| 25 | Mechanical gyro stab. | Compliant | Brass momentum wheel |
| 26 | XBee/LoRaWAN | Compliant | XBee Pro S2C |
| 27 | PANID = team no. | Compliant | Declared at CDR |
| 28 | No broadcast mode | Compliant | Point-to-point |
| 29 | No interference | Compliant | Channel/PANID plan |
| 30 | Own GS, SI plots | Compliant | PyQt SI GUI |
| 31 | GS commands TX | Compliant | START_TX |
| 32 | Elevated antenna | Compliant | Yagi tripod |
| 33 | GS stability | Compliant | Tripod |
| 34 | No TX until cmd | Compliant | Gated on ARM |
| 35 | GS calibration cmd | Compliant | CAL zeroing |
| 36 | GS CSV | Compliant | Flight_<ID>.csv |
| 37 | 1s time resolution | Compliant | millis() + DS3231 RTC |
| 38 | Survive reset | Compliant | Flash NVS + RTC wall-clock |
| 39 | GS laptop+XBee+ant | Compliant | Section 9 |
| 40 | Portable GS | Compliant | Mobile |
| 41 | Telemeter state | Compliant | Field 15 (states 0-7) |
| 42 | ASCII CSV >=1Hz | Compliant | CR-terminated 1Hz |

## 14.1 Optional Objectives Claimed (extra weightage, Rule 3.2)

| Objective | Status | How met |
| --- | --- | --- |
| (i) Novel descent trajectory control | Claimed | GPS-guided steerable cruciform main canopy |
| (ii) Additional innovative sensors | Claimed | 6-sensor atmospheric pollution payload |
| (iii) Video separation -> touchdown | Claimed | Onboard action camera to own microSD |
| (iv) Controlled flight & targeted landing | Claimed | Homes to launch-pad waypoint; flare for soft landing |
| NavIC / Made-in-India weightage | Claimed | NavIC receiver; India-sourced structure/PCB/mechanics |

— END OF DOCUMENT — Version 5.0 — Payload + Targeted Landing Edition —