PRELIMINARY DESIGN REVIEW

(PDR) DOCUMENT

IN-SPACe CAN-7USAT India Student Competition 2026 (3rd Edition)

Atmospheric Pollution & Wildfire-Smoke Profiling CAN-7USAT

| Team Name | [ TEAM TO FILL ] |
| --- | --- |
| Team Identification No. | 2026-IN-SPACe-CAN-7USAT-XXX [ assigned by organizers ] |
| Institution | Bennett University |
| Club / Group | Astronomy Club — Space Tech Division |
| Faculty Advisor | [ TEAM TO FILL ] |
| Document | Preliminary Design Review (PDR) |
| Submission Window | 16-30 May 2026 |
| Reference | Mission Requirements Doc. IN-SPACe.CAN.MR.CG.01, Issue 01 |

This PDR is structured to match the official documentation guidelines (Sections 7-11) section-by-section, so the jury can map every requirement directly. Yellow boxes flag where team-specific content (names, CAD renders, photos, simulation plots) must be inserted before submission.

# TABLE OF CONTENTS

(Right-click → Update Field in Word to populate page numbers.)

# LIST OF ACRONYMS

Per the guidelines, all acronyms are listed at the start of both the document and the presentation.

| Acronym | Expansion |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| AGL | Above Ground Level |
| BOM | Bill of Materials |
| BSEC | Bosch Software Environmental Cluster (gas library) |
| CDR | Comprehensive Design Review |
| CG / MI | Centre of Gravity / Moment of Inertia |
| CONOPS | Concept of Operations |
| EPS | Electrical Power Subsystem |
| FRR | Flight Readiness Review |
| GNSS | Global Navigation Satellite System |
| GUI | Graphical User Interface |
| IAQ | Indoor/Index Air Quality |
| IMU | Inertial Measurement Unit |
| I2C / SPI / UART | Serial communication buses |
| LDO | Low-Dropout regulator |
| MOS | Metal-Oxide Semiconductor (gas sensor) |
| NavIC | Navigation with Indian Constellation |
| PCB | Printed Circuit Board |
| PDR | Preliminary Design Review |
| PM | Particulate Matter |
| PVT | Position, Velocity, Time |
| RHCP | Right-Hand Circular Polarization |
| TM / TC | Telemetry / Telecommand |
| VOC | Volatile Organic Compound |

# 1. MISSION REQUIREMENTS & HOW WE INCORPORATE THEM

This CAN-7USAT meets the 2026 mission requirement: under 1 kg, within 0.15 m x 0.40 m, launched to 1000 m, with a descent control system opening at 600 m and mechanical-gyro stabilization, while telemetering all mandatory datasets. On top of the mandatory mission we claim four optional objectives: (i) novel descent trajectory control, (ii) an atmospheric pollution science payload, (iii) full-flight video capture from separation to touchdown, and (iv) controlled flight & targeted landing via a GPS-guided steerable parachute (Rule 3.2).

| Mission Requirement | Our Incorporation |
| --- | --- |
| Mass < 1 kg (+/-50g) | ~710g design, ~290g margin |
| Envelope 0.15m x 0.40m | 148mm dia x 390mm height |
| Launch to 1000m, deploy near apogee | Free-flying CANSAT; drogue drag-deploys at ejection |
| Descent control opens at 600m | GPS-guided steerable main parachute at 600m |
| Mechanical gyroscopic stabilization | Brass momentum wheel on yaw axis, RPM telemetered |
| Mandatory datasets + telemetry | BMP390, BNO055, NavIC GNSS, Hall, ADC, DS3231 RTC; XBee TM, SD backup |
| Optional (i): trajectory control | GPS-guided steerable cruciform main canopy |
| Optional (ii): novel sensors | 6-sensor pollution payload (PM, NO2/CO, CO2, NH3, VOC, UV) |
| Optional (iii): video capture | Onboard action cam records separation -> touchdown to own microSD |
| Optional (iv): targeted landing | Homes to launch-pad waypoint; flare for soft landing |
| Optional: NavIC, Made-in-India | NavIC receiver; India-sourced structure/PCB/battery/mechanics |

# 2. DERIVED & ALLOCATED SYSTEM-LEVEL REQUIREMENTS

| ID | Derived Requirement | Allocated To | Verification |
| --- | --- | --- | --- |
| SYS-01 | Total mass <= 1000g | All subsystems (mass budget) | Weigh-in |
| SYS-02 | Survive 15G accel / 30G shock | Mechanical (FRP shell, hard mounts) | Vibration + drop test |
| SYS-03 | Telemetry >= 1 Hz, all fields | C&DH + Software | Bench + sim test |
| SYS-04 | Primary descent 20+/-5 m/s | Descent (drogue sizing) | Drop test + sim |
| SYS-05 | Secondary 1-3 m/s + targeted landing @600m | Descent (steerable main + winch servos) | Drop test + guided drop |
| SYS-06 | Attitude stabilization | Momentum wheel | Spin test, RPM telemetry |
| SYS-07 | 2h pad + flight power | EPS (3500mAh + heater mgmt) | Endurance test |
| SYS-08 | Data survives link loss | Software (SD logging) | Sim + read-back |
| SYS-09 | Recovery aid >= 92 dB | Mechanical (beacon) | SPL meter |
| SYS-10 | Pollution profile capture | Payload (6 sensors) | Calibration + sim |

# 3. OPERATIONAL ASPECTS (CONOPS)

PRE-LAUNCH        Assemble, power ON (ext switch+LED), GS ARM,

CAL zero baro/gyro/accel, GNSS fix, HEATER_ON

~30 min before launch (payload warm-up).

ASCENT            Rocket to ~1000m; CANSAT logs, awaits ejection.

EJECTION (1000m)  Drogue drag-deploys; momentum wheel spins up.

DESCENT-1         ~20 m/s to 600m; full TM + payload + SD.

AEROBREAK (600m)  Steerable main releases; descent -> 1-3 m/s.

GUIDED DESCENT    GPS homing to launch pad; dense pollution sampling.

TOUCHDOWN         92dB beacon; surface readings; GNSS coords TX.

RECOVERY          Team homes on beacon + GPS; retrieve; PFR data.

[ TEAM TO FILL ] Insert a CONOPS graphic (rocket -> apogee -> drogue -> main at 600m -> landing) with altitudes labeled. A simple annotated diagram is expected here.

# 4. TEAM COMPOSITION & MANAGEMENT

Team of 5-8 undergraduate members with one faculty advisor (and up to two mentors). The team leader presents the PDR.

## 4.1 Team Roster

[ TEAM TO FILL ] Complete one row per member. Roles must map to subsystems (Mechanical, Avionics/PCB, Software, Comms/GS, Payload, Systems/PM).

| Name | Course / Year | Specialization | Role in Team |
| --- | --- | --- | --- |
| [ Member 1 - Team Leader ] | B.Tech ECE, 2027 | Embedded / Systems | Team Lead, Avionics & Integration |
| [ Member 2 ] | [ course/year ] | [ area ] | Mechanical & Structure |
| [ Member 3 ] | [ course/year ] | [ area ] | Flight Software & Algorithm |
| [ Member 4 ] | [ course/year ] | [ area ] | Communications & Ground Station |
| [ Member 5 ] | [ course/year ] | [ area ] | Payload (Sensors) & Calibration |
| [ Member 6-8 ] | [ course/year ] | [ area ] | Power / Testing / Recovery |
| [ Faculty Advisor ] | Faculty | [ dept ] | Advisor (oversight role only) |

## 4.2 Organization & Management Approach

- Weekly progress meetings; tasks tracked against the milestone chart (Section 16).

- Subsystem leads own design + testing of their subsystem and report to the team lead.

- All correspondence with organizers routed through the faculty advisor (per rules).

- Version-controlled design docs and firmware (Git); software change log maintained.

## 4.3 Financials (BOM & Costing Summary)

Detailed BOM follows in Section 15. Summary of cost categories required by the guidelines:

| Cost Category | Estimate (Rs.) | Actual (Rs.) | Deviation |
| --- | --- | --- | --- |
| Components (electronics + payload) | 23,770 | [ fill ] | [ fill ] |
| Ground station (laptop owned) | 0 | [ fill ] | [ fill ] |
| Parachute (drogue + main + lines) | 1,400 | [ fill ] | [ fill ] |
| Designing | 0 (in-house) | [ fill ] | [ fill ] |
| Prototyping (PCB, 3D prints) | 2,000 | [ fill ] | [ fill ] |
| Fabrication (composite layup) | 7,400 | [ fill ] | [ fill ] |
| Testing (consumables) | 1,500 | [ fill ] | [ fill ] |
| Contingencies (15%) | 5,325 | [ fill ] | [ fill ] |
| Second-hand hardware | None | [ fill ] | [ fill ] |
| TOTAL | ~40,823 | [ fill ] | [ fill ] |

FUNDING NOTE IN-SPACe does NOT provide project funding. Max CAN-7USAT cost is Rs. 1,00,000 (declared at CDR). Sponsors may contribute up to Rs. 2,00,000 (excl. travel). List any sponsors here.

[ TEAM TO FILL ] List sponsors / sponsorship status, if any.

# 5. MISSION OVERVIEW

## 5.1 Mission Objectives

Primary (mandatory): Telemeter position, altitude, pressure, temperature, orientation, power and system status in real time; execute two-stage descent control; stabilize attitude with a mechanical gyroscopic wheel; recover safely.

Secondary / Add-on (bonus): Build a vertical atmospheric pollution profile of the air column (PM1.0/2.5/10, NO2, CO, CO2, NH3/benzene, VOC index, UV) for urban air-quality and wildfire-smoke applications. Provide NavIC navigation and maximize Made-in-India content.

## 5.2 System-Level Requirements Overview

See Section 2 for the derived requirements. The system fits the mass/volume/budget envelope, survives launch loads, and satisfies all 42 compliance requirements (Section 17).

## 5.3 External Support

[ TEAM TO FILL ] State any external facilities used beyond the institute (e.g., a partner lab for composite layup, vacuum chamber, vibration table, antenna range). If all in-house, state so.

## 5.4 System Overview

### 5.4.1 CAN-7USAT Configuration

___________________________

/                           \   GNSS antenna (zenith)

|  TOP CAP + GNSS ANTENNA     |   15mm

|  PRIMARY DROGUE (passive)   |   40mm

|== STEERABLE MAIN + STEERING =|   65mm  -- GPS-guided @600m

|== MOMENTUM WHEEL (gyro) ====|   40mm

|== AVIONICS + ENV (clean) ===|   45mm  ESP32/BMP390/BME688/

|                             |        BNO055/SCD41/uSD

|== PARTICULATE BAY (SPS30) ==|   30mm  ducted airflow

|== HEATED-GAS (MQ135/MiCS) ==|   35mm  + UV, isolated

|== COMMS (XBee + NavIC) =====|   30mm

|== POWER (18650 + regs) =====|   70mm

|  BOTTOM CAP + bumper        |   15mm

\___________________________/

148mm dia | ~390mm | ~710g

### 5.4.2 Major Components

- Structure: vacuum-bagged fiberglass/epoxy shell (RF-transparent) + PLA+ bulkheads.

- Avionics: ESP32-WROOM-32 on a custom 2-layer PCB.

- Descent: drogue (passive) + GPS-guided steerable main + brass momentum wheel.

- Comms: XBee Pro S2C (2.4 GHz) + Yagi ground station.

- Payload: BME688, MiCS-4514, SPS30, SCD41, MQ-135, GUVA-S12SD.

- Imaging: onboard action camera (full-flight video to own microSD).

### 5.4.3 Engineering Drawings

[ TEAM TO FILL ] Insert CAD: (a) isometric exploded view, (b) dimensioned side section, (c) PCB layout, (d) momentum-wheel assembly, (e) parachute attachment/swivel detail. Use Fusion 360 / SolidWorks / FreeCAD.

### 5.4.4 Launch & Descent Strategy

Drogue deploys passively at ejection (20 m/s); a GPS-guided steerable main releases at 600m (1-3 m/s) and homes toward the launch-pad waypoint; momentum wheel stabilizes attitude throughout; dense pollution sampling occurs during the slow guided descent. See the dedicated Parachute & Descent Subsystem document for full guidance detail.

### 5.4.5 Post-Launch Recovery

92 dB audio beacon activates at touchdown; the team homes in using the beacon and live GNSS coordinates. Tri-lingual labeling (English/Hindi/regional) and fluorescent orange body aid visual recovery.

### 5.4.6 Data Retrieving & Analysis

Telemetry is plotted live at the ground station and saved as Flight_<TEAM_ID>.csv. Onboard SD provides a backup. Post-flight, vertical profiles (each pollutant vs altitude) are generated for the PFR.

# 6. SUBSYSTEM DETAILS

## 6.1 Payload Subsystem

The payload is the atmospheric pollution sensor suite. It has no deployable mechanism (sensors sample ambient air through screened vents and a ducted PM inlet), so it adds no mechanical deployment risk.

| Sensor | Interface | Measures (pollution role) | Resolution | Mass / Cost |
| --- | --- | --- | --- | --- |
| BME688 | I2C 0x76 | VOC, humidity, T, P (air-quality baseline) | IAQ index | 2g / Rs.1200 |
| MiCS-4514 | 2x analog | NO2 + CO (vehicle exhaust) | rel ppb/ppm | 5g / Rs.1500 |
| SPS30 | I2C 0x69 | PM1.0/2.5/10 (haze, soot, ash) | ug/m3 | 26g / Rs.4000 |
| SCD41 | I2C 0x62 | CO2 (combustion/greenhouse) | ppm | 5g / Rs.2500 |
| MQ-135 | analog | NH3, benzene (industrial/agri) | rel | 15g / Rs.250 |
| GUVA-S12SD | analog | UV irradiance | UV index | 1g / Rs.250 |

### 6.1.1 Electrical & Mechanical Interfaces

- Electrical: 5 I2C devices share one bus (distinct addresses); 5 analog outputs to ESP32 ADC1; gas heaters gated by a MOSFET on GPIO13.

- Mechanical: SPS30 flat-mounted in a ducted bay (inlet top, outlet bottom); MQ-135/MiCS in a separate thermally-isolated vented bay away from BMP390/SCD41.

### 6.1.2 Design Constraints & Trade-offs (Mass / Power / Volume)

| Constraint | Trade-off Made |
| --- | --- |
| Volume (bulky PM + camera) | Camera mounted radially (lens at side window); tight repack to ~390mm |
| Power (heater load) | Heaters gated ON ~30 min pre-launch, not full 2h pad wait |
| Thermal (heater heat) | Isolated heated-gas bay; precision sensors in clean-air bay |
| Accuracy (MOS sensors) | Report relative profiles + calibration curves, not absolute ppm |
| Made-in-India vs science | Accepted import sensors for the innovation bonus |

## 6.2 Housekeeping Subsystem

The housekeeping ('satellite bus') comprises the Mechanical, Communication & Data Handling, Electrical Power, and Sensor subsystems, detailed in Sections 7-10.

# 7. MECHANICAL SUBSYSTEM

| Structure | Vacuum-bagged E-Glass/epoxy sandwich shell (RF-transparent) |
| --- | --- |
| Bulkheads / mounts | PLA+ rings (7), screwed to shell; electronics on M3 standoffs |
| Material selection | Fiberglass (not carbon) for RF transparency; foam core for stiffness |
| Thermal management | Isolated heated-gas bay; vents for sensors; clean-air bay for baro/CO2 |
| Launch loads | Designed for 15G sustained + 30G shock |
| Interfaces | Top-cap chute swivel; servo release; ducted PM airflow path |
| Color / labeling | Fluorescent orange + Indian flag; tri-lingual team ID/contact |

## 7.1 Design & Simulations

[ TEAM TO FILL ] Insert: (a) static structural FEA of the shell under 30G (stress/displacement plots), (b) CG & MI computation, (c) drop-impact estimate. Tools: Fusion 360 Simulation / Ansys / SolidWorks Simulation.

## 7.2 Methodology

- 3D-print PLA mandrel -> wet layup E-Glass + foam core -> vacuum bag cure -> demold -> trim and drill vents/ports.

- 3D-print bulkheads and brackets; integrate bays bottom-up; verify fit at each stage.

## 7.3 Subsystem Placement & Mass Statement

| Item | Mass (g) |
| --- | --- |
| Payload (6 sensors + mount/ducting) | 54 |
| Container/structure (shell + caps + 7 bulkheads + brackets) | 245 |
| Sensors other than payload (BMP390, BNO055, GNSS, Hall) | 27 |
| Parachutes (drogue + main + lines + release) | 68 |
| Momentum wheel assembly | 80 |
| Power (18650 + sled + regs) | 75 |
| Comms + SD + beacon + switch + wiring | 68 |
| TOTAL | ~710 |

# 8. COMMUNICATION & DATA HANDLING SUBSYSTEM

| Radio (XBee selection) | XBee Pro S2C, 2.4 GHz, point-to-point (no broadcast), PANID = team no. |
| --- | --- |
| TxRx antenna | Onboard: quarter-wave whip along body axis (omnidirectional, 2.2-4 dBi). Ground: Yagi-Uda (10-14 dBi) on tripod |
| Antenna material | Whip: tinned steel/brass wire; aligned to CANSAT cylinder axis |
| Link margin | [ TEAM TO FILL — compute from TX power +18dBm, path loss at 1km, RX sensitivity, antenna gains ] |
| Processor type | ESP32-WROOM-32, 240 MHz dual-core |
| Boot time | < 2 s to telemetry-ready |
| Data interfaces | I2C, UART x2, SPI, ADC, PWM |
| Memory / storage | 4MB flash; MicroSD (SPI) backup logging |
| Real-time clock | DS3231 battery-backed RTC + millis() mission time, persisted to flash (survives reset) |
| Transmission control | TM start/stop + calibration via RF command; TX only after ARM |
| Backup on TM loss | Identical CSV logged to MicroSD (Rule 5.1-xi) |
| Data format | ASCII CSV + carriage return, >= 1 Hz |

[ TEAM TO FILL ] Insert the link-budget table (TX power, cable/connector loss, antenna gains, free-space path loss at 1000m for 2.4GHz, RX sensitivity, resulting margin in dB). Target margin > 10 dB.

# 9. ELECTRICAL POWER SUBSYSTEM

| Source | 1x 18650 Li-ion, 3.7V, 3500 mAh (metal can; LiPo prohibited) |
| --- | --- |
| Connection type | Single cell; soldered tabs / spot-welded nickel (NO spring contacts) |
| Regulation | AMS1117-3.3 (3.3V logic/sensors) + 3A boost (5V motor/fan/heaters/servo) |
| External switch + indicator | Body-mounted toggle WITH indicator LED + power-on buzzer chirp (Rule 5.3-i: light OR sound, both provided) |
| Audio beacon | 95 dB(A) siren, exceeds 92 dB minimum; continuous after landing (Rule 5.2-v) |
| Battery access | Removable screw-in sled; swap < 1 minute |
| Heater management | MQ-135 + MiCS heaters gated by MOSFET (RF HEATER_ON ~30 min pre-launch) |
| Monitoring | Voltage divider (2x100k) to ADC; telemetered at 0.01V |

## 9.1 Power Connection Schematic

[ TEAM TO FILL ] Insert EPS schematic: 18650 -> switch -> [3.3V LDO rail] and [5V boost rail] -> labeled loads; show voltage divider and heater MOSFET. Note: single cell (no series/parallel pack).

## 9.2 Power Budget (Energy Balance)

| Phase | Avg Current | Duration | Energy |
| --- | --- | --- | --- |
| Pad wait (heaters OFF) | ~500 mA | 1.5 h | 0.75 Ah |
| Pre-launch warm-up (heaters ON) | ~1050 mA | 0.5 h | 0.53 Ah |
| Flight (all ON incl. gyro) | ~1050 mA | ~0.17 h | 0.18 Ah |
| TOTAL required | - | ~2.2 h | ~1.46 Ah |
| Capacity available | - | - | 3.5 Ah |
| Margin | - | - | >2x |

# 10. SENSOR SUBSYSTEM

Summary of all sensors with interface, resolution, weight, cost and reason for selection.

| Sensor (function) | Interface | Resolution | Wt/Cost | Reason for Selection |
| --- | --- | --- | --- | --- |
| NavIC GNSS (position) | UART | 0.0001 deg / 0.1m | 20g/Rs.3500 | NavIC = extra points; PVT data |
| BMP390 (altimetry) | I2C | 0.1 m | 2g/Rs.900 | High-precision baro altitude |
| BMP390 (pressure) | I2C | 1 Pa | (same) | Shared with altimetry |
| BMP390/BME688 (temp) | I2C | 0.1 C | (same) | Cross-checked T |
| BNO055 (orientation/accel) | I2C | deg, m/s2 | 3g/Rs.1400 | On-chip fusion; absolute orientation |
| A3144 Hall (gyro spin rate) | Digital | deg/s | 1g/Rs.40 | Momentum-wheel RPM (mandatory field 14) |
| Divider->ADC (power status) | Analog | 0.01 V | ~0g/Rs.10 | Bus voltage monitoring |
| SPS30 (PM) | I2C | ug/m3 | 26g/Rs.4000 | Laser PM for smog/wildfire |
| MiCS-4514 (NO2/CO) | Analog | rel | 5g/Rs.1500 | Traffic-exhaust tracer |
| SCD41 (CO2) | I2C | ppm | 5g/Rs.2500 | True photoacoustic CO2 |
| MQ-135 (NH3/benzene) | Analog | rel | 15g/Rs.250 | Industrial/agri off-gassing |
| GUVA-S12SD (UV) | Analog | UV index | 1g/Rs.250 | UV-vs-altitude science |
| DS3231 (RTC) | I2C 0x68 | UTC time | 5g/Rs.150 | Battery-backed clock; SD+TM timestamps |
| Action camera (opt iii) | 5V/GPIO15 | video | 14g/Rs.2800 | Full-flight video to own microSD |

## 10.1 Sensor Trade-offs

- Altimetry: BMP390 chosen over BMP280/MS5611 for best altitude precision and drone/rocket heritage.

- Orientation: BNO055 chosen over raw MPU9250 to offload sensor fusion and guarantee stable orientation under vibration.

- PM: SPS30 over PMS5003 (lighter, I2C, higher quality) despite higher cost.

- Gas (MOS): MQ-135/MiCS accepted for coverage of NH3/benzene/NO2/CO that digital chips miss; limitation is relative (not absolute) readings.

# 11. CAN-7USAT ALGORITHM DESCRIPTION

| Architecture | Single-loop super-loop with interrupt-driven Hall RPM + timed sensor reads |
| --- | --- |
| Language | C++ (Arduino-ESP32 core) |
| Dev environment | PlatformIO / Arduino IDE; Git version control + change log |
| Sampling rate | >= 1 Hz telemetry; sensors polled at 1-5 Hz; SCD41 ~0.2 Hz; SPS30 1 Hz |

## 11.1 State Machine & Flow

[0 BOOT] init sensors/SD/GNSS; restore time+state if reset

v

[1 TEST_MODE] sim profile (GS-commanded)

v

[2 LAUNCH_PAD] ARM; CAL zeroing; HEATER_ON pre-launch

v  (accel > 3G)

[3 ASCENT] log; await ejection

v  (ejection detected)

[4 ROCKET_DEPLOY] drogue drag-deploy; spin up wheel

v

[5 DESCENT] ~20 m/s; full TM + payload + SD

v  (alt <= 600m)

[6 AEROBREAK_RELEASE] steerable main -> 1-3 m/s; GPS guidance ON

v  (landed)

[7 IMPACT] beacon ON; surface readings; GPS TX; finalize CSV

## 11.2 Required Software Details

- Software sequencing/timing diagram: [ TEAM TO FILL — timeline of task execution per loop ].

- Telemetry & telecommand: TM packet @1Hz; TC = START_TX/STOP_TX/CAL/HEATER/SIM.

- Data storage & handling: dual write (XBee + MicroSD); CSV with header.

- Software reset loop: state + mission time persisted to NVS flash; restored on boot.

- Simulation mode strategy: SIM_ENABLE then SIM_ACTIVATE; GS streams altitude profile; software runs full state machine on simulated data.

- Test methodology + proto testing: bench reads -> HIL sim -> drop tests -> full mission sim (Section 13).

[ TEAM TO FILL ] Insert the actual algorithm flowchart (boxes/arrows) and a software sequencing timing diagram.

# 12. GROUND STATION

| Hardware | Laptop (>=2h battery), XBee Pro S2C on USB, Yagi antenna on tripod |
| --- | --- |
| Power & duration | Laptop battery + power bank; >2h operation |
| Antenna type / pattern | Yagi-Uda directional, 10-14 dBi; rotatable for polarization alignment |
| Software | Python (PyQt5 + pyqtgraph) |
| Telemetry display | All 16 mandatory + payload fields, SI units, each plotted live |
| Command interface | Buttons: START_TX, STOP_TX, CAL, HEATER_ON/OFF, SIM_ENABLE/ACTIVATE |
| Real-time scheme | Serial read thread -> parse CSV -> update plots + map @1Hz |
| Last-command display | Console shows last TC sent + last packet received |
| CSV output | Auto-writes Flight_<TEAM_ID>.csv with headers |
| Libraries | pyserial, PyQt5, pyqtgraph, pandas, folium (map) |
| Comm test plan | Range test to 1.5km LOS; packet-loss measurement |

[ TEAM TO FILL ] Insert a ground-station GUI mock-up/screenshot (plots layout + command panel + map + console) and the antenna setup photo/diagram.

# 13. INTEGRATION & TESTING

## 13.1 Subsystem Test Plans

| Subsystem | Test | Pass Criteria |
| --- | --- | --- |
| Payload & sensors | Bench read + calibration + smoke/CO2 exposure | Valid data; repeatable response |
| Mechanical structure | Drop + vibration + housing fit | No damage; clears fairing |
| Command & data handling | TM @1Hz; TC; SD logging | All fields; no loss; CSV intact |
| Power supply | Endurance with heater mgmt | 2h pad + flight met |
| Ground station | Live display + CSV + range | >95% packets @1.5km |
| Algorithm | Full mission sim | All 8 states; correct transitions |

## 13.2 Required Tests

- Drop test, Thermal test, Vibration test, Housing fit-check test.

- Integrated functional test, Environmental test, Simulation test.

- Parachute deployment test (drogue drag + main servo release).

- Polarity test (verify sensor sign conventions vs CANSAT axes).

[ TEAM TO FILL ] As tests are completed, insert photos + result plots (drop, vibration, parachute deploy, gyro RPM, mission sim CSV).

# 14. MISSION OPERATIONS & ANALYSIS

## 14.1 Launch-Day Sequence of Events

T-60min  Arrive, inspect, weigh-in, mount antenna, boot GS

T-40min  Power CANSAT (switch+LED), GNSS fix, CAL zeroing

T-30min  HEATER_ON (payload warm-up), verify all TM fields

T-10min  Integrate into rocket payload bay, fold chutes

T-0      Launch; monitor ASCENT telemetry

Apogee   Ejection; confirm drogue + wheel spin-up via TM

600m     Confirm steerable main; GPS homing; descent 1-3 m/s

Land     Beacon ON; record GPS; begin recovery

Post     Retrieve SD; export CSV; build profiles for PFR

## 14.2 Recovery, Data Analysis, Color-Coding

- Recovery: home on 95 dB audio beacon (exceeds 92 dB minimum) + power-indicator light + live GNSS coordinates; fluorescent body for visual ID.

- Data analysis: vertical profiles (each pollutant vs altitude); descent-rate verification; gyro stability vs IMU.

- Color-coding: [ TEAM TO FILL — wiring/connector color scheme + bay labeling convention ].

## 14.3 Antenna & Ground Station Setup

Yagi on tripod, elevated and aimed along the expected flight line; polarization aligned with the onboard whip; team positioned to relocate if landing drifts.

# 15. BUDGET DETAILS (FULL BOM)

## 15.1 Core Electronics

| Item | Qty | Est. Rs. | Origin |
| --- | --- | --- | --- |
| ESP32-WROOM-32 | 1 | 500 | Import |
| BMP390 | 1 | 900 | Import |
| BNO055 | 1 | 1,400 | Import |
| NavIC GNSS + antenna | 1 | 3,500 | India (pref.) |
| A3144 Hall + magnet | 1 | 40 | India |
| XBee Pro S2C x2 + antennas | 2 | 4,500 | Import |
| XBee USB explorer | 1 | 500 | Import |
| MicroSD module + 16GB card | 1 | 400 | India |
| 18650 3500mAh + sled + regs | 1 | 720 | India |
| 95dB beacon + switch + LED | 1 | 160 | India |
| Custom PCB (5 pcs) | 1 | 600 | India |
| Connectors/passives/MOSFETs | set | 500 | India |

Core subtotal: ~Rs. 13,720

## 15.2 Atmospheric Payload

| Item | Qty | Est. Rs. | Origin |
| --- | --- | --- | --- |
| BME688 | 1 | 1,200 | Import |
| MiCS-4514 | 1 | 1,500 | Import |
| Sensirion SPS30 | 1 | 4,000 | Import |
| SCD41 | 1 | 2,500 | Import |
| MQ-135 | 1 | 250 | Import |
| GUVA-S12SD | 1 | 250 | Import |
| Mounting + ducting | set | 350 | India |

Payload subtotal: ~Rs. 10,050

## 15.3 Mechanical, Descent & Structure

| Item | Qty | Est. Rs. | Origin |
| --- | --- | --- | --- |
| Brass momentum wheel + motor + bearings | set | 630 | India |
| Steering winch servos x2 + lines | set | 1,650 | RC supplier |
| DS3231 RTC + CR2032 | 1 | 150 | India |
| Action camera + mount | 1 | 3,000 | Import |
| Main + drogue parachute + lines + swivel | set | 1,400 | India |
| Release servo + pin + spring | set | 300 | Import/India |
| Fiberglass + foam + epoxy | set | 3,400 | India |
| PLA+ filament | 1 | 1,398 | India |
| Layup consumables + PPE | set | 4,000 | India |
| Paint/flag/fasteners | set | 600 | India |

Mechanical subtotal: ~Rs. 11,728

## 15.4 Grand Total

| Category | Rs. | % of Rs.1,00,000 cap |
| --- | --- | --- |
| Core electronics | 13,720 | 13.7% |
| Atmospheric payload | 10,050 | 10.1% |
| Mechanical + structure | 11,728 | 11.7% |
| Contingency (15%) | 5,325 | 5.3% |
| TOTAL BUILD | 40,823 | 40.8% |

# 16. DEVELOPMENT SCHEDULE (MILESTONE CHART)

| Milestone | Start | End | Owner |
| --- | --- | --- | --- |
| Registration + team finalize | Apr 2026 | 30 Apr 2026 | Lead/Advisor |
| PDR document + PPT | May 2026 | 30 May 2026 | All |
| Procurement (order long-lead sensors) | May 2026 | Jun 2026 | Payload |
| MOS sensor burn-in + calibration | Jun 2026 | Jul 2026 | Payload |
| Structure layup + 3D prints | Jun 2026 | Jul 2026 | Mechanical |
| PCB design + fab + assembly | Jun 2026 | Jul 2026 | Avionics |
| Firmware + ground station | Jun 2026 | Aug 2026 | Software/Comms |
| Subsystem testing | Jul 2026 | Aug 2026 | All |
| CDR document + submission | Jul 2026 | 31 Jul 2026 | All |
| Integration + environmental tests | Aug 2026 | Sep 2026 | All |
| Flight Readiness Review | Sep 2026 | 15 Oct 2026 | All |
| Launch + PFR (Kushinagar) | 16 Oct 2026 | 30 Nov 2026 | All |

[ TEAM TO FILL ] Convert this to a Gantt chart (bars across a month timeline) for the PDR — visual milestone charts score better than tables.

# 17. REQUIREMENTS COMPLIANCE STATEMENT

The design complies with all 42 requirements in the official compliance matrix. Summary of the most safety-critical items:

| Requirement | Compliance |
| --- | --- |
| Mass < 1kg / envelope 0.15x0.40m | ~710g / 148x390mm |
| Two descent mechanisms; primary passive at ejection | Drogue (drag) + steerable main (@600m) |
| Primary 20+/-5 m/s; secondary 1-3 m/s | ~25cm cruciform drogue; ~2.0m steerable main |
| Mechanical gyro stabilization + spin telemetry | Brass momentum wheel + Hall RPM |
| No pyrotechnics; no LiPo; no spring battery contacts | Servo/mechanical; 18650; soldered tabs |
| Survive 15G/30G; hard-mounted; enclosed | FRP shell + standoffs; sensors vented |
| XBee/LoRaWAN; no broadcast; PANID=team; RF cmd | XBee Pro S2C, point-to-point |
| Own GS, SI real-time plots, CSV, calibration cmd | PyQt GUI; CAL; Flight_<ID>.csv |
| Beacon >=92dB; tri-lingual labels; fluorescent+flag | 95dB; Eng/Hindi/regional; orange+flag |
| SD backup; mission time survives reset | MicroSD; flash-persisted clock/state |

The complete 42-row compliance matrix (mapping every requirement to its implementation) is maintained and presented in full in the appendix and CDR.

# ANNEXURE — REFERENCES

- IN-SPACe CAN-7USAT India Student Competition 2026 (3rd Edition): Mission Requirement & Competition Guidelines, Doc. IN-SPACe.CAN.MR.CG.01, Issue 01, 24-03-2026.

- Balanis, C., Antenna Theory: Analysis and Design, 3rd ed., Wiley-Interscience, 2005.

- Kraus J.D., Marhefka R.J., Antennas for All Applications, 3rd ed., McGraw-Hill, 2002.

- Bosch Sensortec datasheets: BMP390, BME688, BNO055.

- Sensirion datasheets: SPS30, SCD41.

- SGX Sensortech MiCS-4514 datasheet; Winsen MQ-135 datasheet.

- [ TEAM TO FILL — any additional papers, app notes, or tools referenced ]

— END OF PDR DOCUMENT —