#include <ESP32Servo.h>

#include <Preferences.h>

#include <Wire.h>

#include <Adafruit_BMP3XX.h>

#define SERVO_PIN 25

#define DEPLOY_ALTITUDE 600.0f

#define BACKUP_DEPLOY_TIME 25.0f

#define LOCKED_ANGLE 0

#define RELEASE_ANGLE 90

#define ALTITUDE_SAMPLES 5

Servo chuteServo;

Adafruit_BMP3XX bmp;

Preferences prefs;

bool mainReleased = false;

bool bmpWorking = false;

float altitudeBuffer[ALTITUDE_SAMPLES];

int altitudeIndex = 0;

float currentAltitude = 0;

float previousAltitude = 0;

float averagedAltitude = 0;

float descentRate = 0;

unsigned long deployStartTime = 0;

unsigned long previousSampleTime = 0;

int frozenCounter = 0;

float groundPressure = 1013.25;

void saveDeploymentState()

{

prefs.begin("flight", false);

prefs.putBool("released", mainReleased);

prefs.end();

}

void restoreDeploymentState()

{

prefs.begin("flight", true);

mainReleased =

prefs.getBool("released", false);

prefs.end();

}

void deployMainParachute()

{

if(mainReleased)

return;

Serial.println("MAIN DEPLOYMENT START");

mainReleased = true;

saveDeploymentState();

chuteServo.write(RELEASE_ANGLE);

delay(1000);

Serial.println("MAIN PARACHUTE RELEASED");

}

float getAverageAltitude(float newAltitude)

{

altitudeBuffer[altitudeIndex] = newAltitude;

altitudeIndex++;

if(altitudeIndex >= ALTITUDE_SAMPLES)

altitudeIndex = 0;

float sum = 0;

for(int i = 0; i < ALTITUDE_SAMPLES; i++)

{

sum += altitudeBuffer[i];

}

return sum / ALTITUDE_SAMPLES;

}

bool sensorFrozen()

{

static float lastAltitude = 0;

if(abs(currentAltitude - lastAltitude) < 0.05)

{

frozenCounter++;

}

else

{

frozenCounter = 0;

}

lastAltitude = currentAltitude;

return frozenCounter > 10;

}

void setup()

{

Serial.begin(115200);                                         //baud rate

restoreDeploymentState();

chuteServo.attach(SERVO_PIN);

if(mainReleased)

{

chuteServo.write(RELEASE_ANGLE);

}

else

{

chuteServo.write(LOCKED_ANGLE);

}

bmpWorking = bmp.begin_I2C();

if(bmpWorking)

{

Serial.println("BMP390 Initialized");

// Take initial pressure reading

if(bmp.performReading())

{

groundPressure =

bmp.pressure / 100.0;

Serial.print(

"Ground Pressure: ");

Serial.print(

groundPressure);

Serial.println(" hPa");

}

}

else

{

Serial.println("BMP390 Failed");

Serial.println("Using Backup Timer Deployment");

}

for(int i = 0; i < ALTITUDE_SAMPLES; i++)

{

altitudeBuffer[i] = 0;

}

deployStartTime = millis();

previousSampleTime = millis();

Serial.println("Deployment System Ready");

}

void checkDeploymentConditions()

{

if(mainReleased)

return;

float elapsedTime =

(millis() - deployStartTime) / 1000.0;

bool backupTrigger =

elapsedTime >= BACKUP_DEPLOY_TIME;

if(!bmpWorking)

{

if(backupTrigger)

{

deployMainParachute();

}

return;

}

bool altitudeTrigger =

averagedAltitude <= DEPLOY_ALTITUDE;

bool descending =

descentRate < 0;

bool freezeTrigger =

sensorFrozen();

if(

(altitudeTrigger && descending)

||

freezeTrigger

||

backupTrigger

)

{

deployMainParachute();

}

}

void loop()

{

if(bmpWorking)

{

if(bmp.performReading())

{

currentAltitude =

bmp.readAltitude(groundPressure);

averagedAltitude =

getAverageAltitude(currentAltitude);

unsigned long now =

millis();

float dt =

(now - previousSampleTime)

/ 1000.0;

if(dt > 0)

{

descentRate =

(currentAltitude

- previousAltitude)

/ dt;

}

previousAltitude =

currentAltitude;

previousSampleTime =

now;

}

}

checkDeploymentConditions();

delay(200);

}